(Approx. 1111 words)

The AMD A8-7670 Processor: A Review

By Daniel Woodard, Member, The Dayton Microcomputer Association, OH

www.dma1.org

http://www.majorgeeks.com/ les/details/3dmark.html

http://www.geeks3d.com/gputest/

DGW (at) DMA1.org

Would you consider yourself a gamer on a budget? If so, then AMD has a processor for you, the Godavari A8-7670K APU. Built using a 28nm process, the 95 Watt 7670K incorporates a Radeon R7 GPU (graphical processing unit) and fits in an FM2+ motherboard socket. The processor runs at a base speed of 3.6 GHz, and ramps up to 3.9 GHz as needed. The graphics portion of the chip runs at 757 MHz and has 384 stream processors.

The processor might appeal to those who already have an FM2+ based motherboard, or want something similar to a base line video card, but with the ability to upgrade easily. These processors have both the CPU and GPU combined into what AMD is calling an APU, or Accelerated Processing Unit. It might also appeal to those who want to minimize system fan noise, since there is no additional fan on a separate video card. A person who wants to keep total wattage under control (for example, to work with an existing system power supply) might also want one of these, since the combined CPU and graphics power usage is under 100 watts. Another plus is that the APU supports running DDR3 RAM at 2133 speeds, a jump up from some of the earlier models. The K at the end of the name also designates that this is a Black Edition, meaning that hobbyists can tinker around with the clock multiplier of both the CPU and GPU portions of the chip. I personally wanted to upgrade my A4-7300 based system, which I had built in late 2014, mostly because there were a few games my kids play where it did not quite keep up with demands. Otherwise, the A4-7300 had been quite an excellent choice for casual games, multimedia and productivity software for about $45.

I first did some “everyday computing” type comparisons between an A4-7300 dual core processor running at 4 GHz and the A8-7670K. I started out running Hyper Pi, which is a multicore computation of Pi, out to 16 million digits. This took 11 minutes and 43 seconds on the A4, but only 9 minutes and 34 seconds on the A8-7670K.

Next, I took a folder containing a mix of files totaling 454 megabytes and used Winrar to do a compression. It took 4 minutes and 27 seconds on the A4, but 5 minutes and 25 seconds on the A8. The results were rather shocking at first—why would a processor with 4 cores take longer than one with 2 cores? Then I realized that I was using a very, very old version of Winrar—almost ten years old. It was very likely that this version only “saw” the first core on each processor, which would give the 4 GHz A4 an edge. However, as soon as I installed the latest version of Winrar and ran it using the A8, I saw the total time to compress the files drop to only 51 seconds! It was quite the Aha! moment for me—I had never before seen quite such a vivid example of why it is so important to upgrade software to more recent versions. Using the older versions of the software had been causing me to take about five times as long to accomplish the same task.

For the final part of the everyday computing benchmark test, I used Handbrake to compress a 44 minute 720p kid video to h.264/mpeg4. On the A4-7300, this took 28 minutes and 52 seconds, giving an average frame rate per second (fps) of 45.6. Using the A8-7670K, this took only 14 minutes and 40 seconds, giving an average fps of 89.6—converting the file in roughly half the time.

For 3D gaming comparisons, I used the 3DMark Sky Diver test, followed by the Geeks 3D GPU Test suite. The 3DMark Sky Diver test is exactly what it sounds like: a sky diver (glider, really—there are arm winglets like a flying squirrel) going through various scenes, putting the video card under strain to assess its capabilities, and giving an overall numerical score for comparison. For the Sky Diver test, final results for the A4 were 2,126 and for the A8 were 4,294—roughly double the ability. I also tested a Pentium G3220 based system I have with an actual Radeon R7 240 video card installed, and it finished the Sky Diver test with a score of 4,082. The Radeon R7 240 video card sells for about $60 retail. As you can see from the results, the A8-7670K has a slightly better 3D performance than a stand-alone $60 video card from about 24 months ago offers. That they have managed to build this into a processor and still stay under 100 Watts of power draw is an accomplishment.

I also ran the Geeks 3D GPUTest suite, which offers a batch of different tests. These include something called furmark, tessmark, GiMark, Pixmark, Plot3D, and others. Many of these are a lot of fun to watch, so I would recommend trying it if you like to watch animations, fractals and such. Rather than putting the number results from all of these here, I’ll just summarize by saying that the tests show an average 110% video performance increase from the A4-7300 to the A8-7670K, again more than doubling.

For a real-world gaming test, I used a Steam game called BeamNG.drive which my oldest son has loved playing with for the past year. It is essentially a 3D car physics simulation where players can drive a variety of vehicles through many different environments. On the A8 I obtained roughly 33-35 frames per second in game, while on the A4 the game showed an average of just 17-18 fps. The slower frame rate was still playable, but seemed very jerky in comparison. A person would definitely not want to go back to the slower frame rate after getting used to the better play of the A8-7670K for a few hours.

The A8-7670K is available for roughly $105 to $110, and sometimes under $99 during sales. From reports on line, it appears that overclocks of 4.5 GHz are easily accomplished with a decent heat sink. For folks who aren’t interested in overclocking, this is definitely one of the first processors around the $100 price point that can competently allow 3D gaming at low to medium resolutions. If AMD continues to make advancements on their APU’s, then we might actually see some folks doing budget gaming builds and returning to PC gaming (Steam, etc.) rather than buying game consoles.