(Approx. 1,946 words)

CD-R and DVD+–R Longevity: How Long Will They Last?

By John Langill, Newsletter Editor, Southern Tier Personal Computing Club, NY

August 2013 issue, Rare Bits

jlangil1 (at) stny.rr.com

Although there are today many data storage alternatives, I’m sure that there are many such as me who in the past stored various kinds of information on optical media, CD-Rs in particular. To cite just two examples; I have scanned hundreds of family slides, organized the digital images, and saved them on CD-Rs. Similarly, I did the same with several hundred of digital photos from my two-month visit with my son and daughter-in- law in Australia in 2003. The purpose of doing so was to have a convenient form in which archive the digital images and to share them with other members of the family; while at the same time conserving space on the hard-disk—then a more precious commodity than it is today.

Thinking back 10 or more years, one may recall that a single CD-R then offered a relatively large data storage capacity in a form that could be easily and inexpensively mailed anywhere in the world—something that could not be accomplished via the Internet or with other “portable” media at the time.

While acknowledging that the images stored on these CD-Rs—and others even older—could now be transcribed to another medium, I confess that I’m reluctant to devote the time and effort to doing so at this point. Accordingly, the durability and life-expectancy of the CD-Rs that I created 10, 20, and even 30 years ago, has become an increasing concern.

In the early ‘90s when the first CD-R discs were introduced manufacturers said the media had a data life in excess of 40 years. In the late ‘90s when the first DVD-R discs appeared on the scene producers proclaimed a data life of at least 100 years. However, in the time since their introduction it has been discovered that  these  early  discs  are  susceptible to  media  “rot”  (i.e.,  “bit  rot”)  that  can  eat  your information—audio, video, or data—in as little as two years after it is written. (According to research fairly recently conducted by J. Perdereau, CD-Rs may have an average life expectancy of not more than 10 years—Journal de 20 Heures, March 2008.)

Because CD-R and DVD+–R media is used to archive nearly everything today, it does make one worry; especially if these discs are the only repository in which your precious, and irreplaceable, family memories— photos and movies—as well as vital family, personal, and company data/documents are stored.

So where does the truth lie? Somewhere across the complete spectrum.

Most people who successfully burn a disc believe they have quality media. Unfortunately that only tells you the disc will be compatible (able to be played) in the vast majority of CD or DVD players. More importantly all better quality CD and DVD burners include technology called over burn/under burn protection making “coaster production” a thing of the past. The basic construction of both disc technologies enable you to burn your data in a very precise, very controlled manner.

Test Options

There are only two foolproof ways of proving the data life of the discs you use:

- Write a few CD-Rs or DVD+–Rs, then wait about 25-50 years and check if they still hold the correct data.

- Use a CD/DVD analyzer that is specially designed to retrieve very accurate information about your media and your data after accelerated aging in test chambers where the discs are subjected to excessive temperature and humidity tests.

The first is typically impractical. Nonetheless, from personal experience I can attest to the fact that the first CD-R I ever burned—selections from a vinyl LP album—plays just fine and the music still sounds great 25 years later. However, I have also had some CD-Rs become unplayable in just a matter of months. Fortunately, such occurrences have been few.

The second provides only theoretical limits and doesn’t take into consideration how you use, handle, and store the media. However, even assuming proper handling, temperature and humidity can adversely affect the data-life of even quality media.

Between the CD-R discs produced in the early 1980s and today’s double-layer DVD+-R discs there has been considerable progress in write performance, capacity, quality, and cost.

Following the test procedures of the International Standards Organization (ISO), quality media manufacturers have been able to predict data-life spans ranging from 50-200 years. But keep in mind there are wide differences between low-budget media manufacturers and quality media manufacturers. In addition variations in manufacturing methods, materials and processes/procedures can dramatically affect the data life of the media you use.

Or as auto manufacturers like to say... “Your mileage may vary.”

Understanding Your Discs

It isn’t vital that you understand the construction of CD-R or DVD+–R media to produce a quality disc that can be read years from now any more than you need to understand the internal combustion engine to drive a car. But understanding the difference between quality and cheap media may help you avoid losing family photos or videos later on.

Most people consider DVD+–R discs little more than overgrown CD-Rs but, while they are similar, they are also quite different. In particular, the grooves are narrower and more closely spaced and the structure (pattern) of “pits” and

“lands” is very much smaller with a DVD+–R in order to enable a greater data storage capacity. Precision is very critical.

Writable CD-R and DVD+–R discs start with a piece of polycarbonate substrate into which very precise grooves are molded to guide the tracking of the laser beam. A dye layer is then precisely applied to the substrate followed by a reflective layer and one or more protective layers. A few of the leading media manufacturers have initiated the policy of applying two very resistant layers for added data protection when the discs are used, handled, and stored.

Because of the faster read/write performance users now expect, leading manufacturers have developed new stamper technology for optimum groove (storage area) shape and ultra-precise molding technology. The molding is critical when the media must withstand being rotated at extremely high speeds during the write process—up to 52x for CD-Rs, and 8x to 16x for DVD+–Rs. The engineering margin that was once reserved for manufacturing tolerance has been used for data capacity instead, leaving no tolerance for manufacturing; for these discs to be truly compliant with the Orange Book standard, the manufacturing process must be perfect.

Media Problems

The quality of your media is directly related with the time the media will last without losing information. As you can see there are a number of areas where manufacturers can shave a few cents in the overall cost of the media and areas where production can go amiss to dramatically shorten the data life of your stored information.

There are conflicting claims and consumer beliefs on which media is best for data retention of 30, 50, 100 years—green, gold, or blue dye; and gold or silver reflective layer. It is somewhat immaterial today. Manufacturers of quality writable discs have developed significantly improved, more sensitive and more stable dyes, and better reflective materials that virtually eliminate data loss during high-speed read/write processes and enhance long-term reliability.

CD and DVD rot (i.e., bit rot) is not the problem today that it was with earlier LaserDiscs because the media use different dye technologies to store data and make it much less susceptible to that kind of degradation. The truth is that deterioration arising from delamination and oxidation is the greater problem.

Delamination and oxidation usually occur at the outer edge of the disc and are often the result of the adhesive not being properly applied and cured during the production process. This usually happens when price-oriented manufacturers use equipment that is 2 to 3 generations old and the least expensive materials possible.

When it does happen the laser is unable to read the data on the reflected layer. It is usually caused by:

•    Oxidation when air comes in contact with the reflective layer

•    Galvanic reaction between the layers and coatings

•    Chemical reaction caused by impurities in the disc’s adhesive or aluminum coating.

•    Excessive heat and humidity are known to accelerate and exacerbate delamination and oxidation.

The Real Culprit

If you have purchased quality media from a quality manufacturer, you are still not assured of 50-100 years of data life!

The greatest danger to the data longevity of your personal, family, and business information is you alone;  that  is,  by  the  way  you  handle  and  store  your  discs.  The  environment—temperature and humidity—can stress the materials. Gravity also can bend and stress the discs. Fingerprints and smudges can do more damage than scratches.

But by following a few Do’s and Don’ts you can ensure your precious family and friend pictures, movies, family records, and business files have the maximum data life.

Do not

- Touch the surface of the disc.

- Bend the disc... especially when removing it from its case as this can cause a fine crack to develop at the rim of the hub-hole which will render the disk useless. This is a particular problem with DVDs.

- Store discs horizontally for a long time (years).

- Open a recordable optical disc package if you are not ready to record.

- Expose discs to extreme heat or high humidity.

- Expose discs to rapid temperature or humidity changes.

- Expose recordable discs to prolonged sunlight or other sources of UV light.

- Write or mark in the data area of the disc (the shiny side that the laser “reads”).

- Clean in a circular direction around the disc.

Do

- Handle discs by the outer edge or the center hole.

- Use a nonsolvent-based felt-tip permanent marker to mark the label side of the disc.

- Keep dirt or other foreign matter from the disc.

- Store discs upright (book style) in original jewel cases that are specified for CDs and DVDs.

- Return discs to their jewel cases immediately after use. Because the label side is more delicate and susceptible to damage, I recommend storing any CD or DVD disc label-side down in its jewel case.

- Leave discs in their spindle or jewel case to minimize the effects of environmental changes.

- Remove protective wrap only when you are ready to record data on the disc.

- Store in a cool, dry, dark environment in which the air is clean—relative humidity should be in the range 20% - 50% (RH) and temperature should be in the range 4°C - 20°C (approx. 40 to 70EF).

- Remove dirt, foreign material, fingerprints, smudges, and liquids by wiping with a clean cotton fabric in a straight line from the center of the disc toward the outer edge.

- Dampen the cloth with a lens cleaner to clean your discs. Dry with photo lens tissue. For tough problems use Windex or a similar glass cleaner, diluted dish detergent, or rubbing alcohol. Rinse and dry thoroughly with a lint-free cloth.

- Check the disc surface BEFORE recording.

Reliable Medium

There is a lot of cheap CD-R and DVD+-R media that has marginal quality. For some applications like games, quality isn’t critical. For irreplaceable, vital data like family photos, special events, vacations, and family/friends memories quality does matter. If you are backing up mission-critical data on your home or business computer, quality matters. Then it is important to select a brand of media that will keep your data safe, secure and available for years to come.

Quality and low prices just don’t seem to mix!

The next step to long-term data reliability is to handle and store the media with the respect your data deserves.