(Approx. 380 words)

System Upgrade

By Jim Quinn, Vice President, Under the Computer Hood UG, CA

www.uchug.org

June 2017 issue, Drive Light

1editor101 (at) uchug.org

My Linux computer system drive (SSD) was running out of space.   The SSD was installed February 2015.  The SSD is used as the root for Mint 17.3.  I moved most of the files from my home folders to the data drive which gave me some breathing room while I ordered a new 1TB SSD.

Storage configuration was as follows; 1TB for Linux Mint 17, 4TB for data, and 6TB for backup. The SSD drive is a Kingston Hyper X 3K SSD, 240GB SATA drive (6Gb/s), model # SH103S3/240G. I ordered a SanDisk SSD Plus 960GB as the replacement for my 240GB SSD.

Items needed:

New SSD

USB3 SATA hard drive dock

Clonezilla ISO CD (bootable)

NOTE:  It is highly recommended you make a backup before proceeding, unless you know you can recover all your files.

My idea was to clone the old SSD to the new SSD with Clonezilla by booting the computer with a bootable CD of Clonezilla. It took a while to clone the system drive (installed in my midtower) to the new SSD in the drive dock. This worked just fine. Then I removed the old SSD and installed the new SSD.

The new SSD would not boot; it appeared to be a boot sector problem. Out came my bootable Boot Repair CD. Booted the computer with Boot Repair CD and ran their auto fix option. After it completed, my computer now displayed a boot menu and I was able to boot to Linux Mint 17.3.

Next, I ran Gparted on the new SSD to expand the partition to use the entire space.  Everything is good to go.

Bootable CD’s or thumb drive ISO’s to keep on hand for fixing computers:

- SpinRite – disk utility by Steve Gibson (grc.com) *

- Gparted (http://gparted.org/) *

- Boot Repair Disk (https://sourceforge.net/projects/boot-repair-cd/) *

- Clonezilla (http://clonezilla.org/) *

- ReDo (http://redobackup.org/) *

- Parted Magic (https://partedmagic.com/) *

- System Rescue (http://www.system-rescue-cd.org/) *

- Linux Mint Distro (https://www.linuxmint.com/) *

*both 32 & 64 bit, if available

I prefer Linux Mint Mate as a distro but any Linux distro you like is a great way to boot a computer with the live CD to help diagnose computer hardware issues.