(Approx. 486 words)

Apple Bytes

Laptop Battery Maintenance Procedures

By Norm Kronenberg, Macintosh Director, Hilton Head Island Computer Club

http://hhicc.org

nkron (at) roadrunner.com

This article has been obtained from APCUG with the author’s permission for publication by APCUG member groups; all other uses require the permission of the author (see e-mail address above).

Standard Maintenance

For proper maintenance of a lithium-based battery, it’s important to keep the electrons

in it moving occasionally. Apple does not recommend leaving your portable plugged in all the time. An ideal use would be a commuter who uses her MacBook Pro on the train, then plugs it in at the office to charge. This keeps the battery juices flowing. If on the other hand, you use a desktop computer at work, and save a notebook for infrequent travel, Apple recommends charging and discharging its battery at least once per month. Need a reminder? Add an event to your desktop’s iCal

Calibrate battery

PowerBook G4 (15-inch Double-Layer SD), MacBook (all models), MacBook Pro (all models), and MacBook Pro (17-inch) (all models).

The battery calibration for the PowerBook G4 (15-inch Double-Layer SD) and any model of MacBook or MacBook Pro has been updated because of a new battery released with this computer. With these computers, follow these steps to calibrate your battery:

- Plug in the power adapter and fully charge your PowerBook’s battery until the light ring or LED on the power adapter plug changes to green and the onscreen meter in the menu bar indicates that the battery is fully charged.

- Allow the battery to rest in the fully charged state for at least two hours. You may use your computer during this time as long as the adapter is plugged in.

- Disconnect the power adapter with the computer still on and start running the computer off battery power. You may use your computer during this time. When your battery gets low, you will see the low battery warning dialog on the screen.

- Continue to keep your computer on until it goes to sleep. Save all your work and close all applications when the battery gets very low, before the computer goes to sleep.

- Turn off the computer or allow it to sleep for five hours or more.

- Connect the power adapter and leave it connected until the battery is fully charged again.

New iPod Touch

A Computer in Your Pocket

The iPhone 3G has gotten all of the attention, but the new iPod Touch is a powerful touchscreen computer, with Wi-Fi access to the Internet, but without the cell phone feature or GPS. It is available in 8-, 16- or 32 MB capacities. Connect to the Internet wherever there’s a wireless network. Surf the web and send email from any Wi-Fi hotspot. Get directions, check the weather, follow stocks, watch YouTube, and more.

The 3.5-inch touchscreen display is perfect for surfing the Web, watching movies and playing games.